H.D.  DEGREASER

Safety Data Sheet

Issue Date:  7-Jan-2015

Revision Date:  7-Jan-2015

Version

1

Product Identifier 	  Product Name

Other means of identification 	  SDS #

UN1418

Recommended use of the chemical and restrictions on use

Recommended Use

Cleaning Compound

Details of the supplier of the safety data sheet 	  Supplier Address

SUNLINE LLC

615 Brookhaven Dr.

ORLANDO FL 32803

Emergency Telephone Number 	  Company Phone Number Emergency Telephone (24 hr)

407-896-2129

407-415-8751

Physical State

Liquid

Classification

Hazards Not Otherwise Classified (HNOC)

None

Signal Word Danger

Hazard Statements

Causes severe skin burns and eye damage Causes serious eye Damage

Harmful if swallowed

Harmful to aquatic life with long lasting effects May be corrosive to metals

__ 	__ 	__ 	__ 	_

Page  1 / 7

Skin corrosion/irritation

Category 1A

Serious eye damage/eye irritation

Category 1

Acute Toxicity, Oral

Category 4

Aquatic Toxicity, Chronic

Category 3

2. HAZARDS IDENTIFICATION

1. IDENTIFICATION

HD  DEGREASER

Revision Date:  7-Jan-2015

__ 	__ 	__ 	__ 	_

Precautionary Statements - Prevention

Wash face, hands and any exposed skin thoroughly after handling Do not eat, drink or smoke when using this product.

Do not breathe dust/fume/gas/mist/vapors/spray

Wear protective gloves/protective clothing/eye protection/face protection Avoid release to the environment

Keep only in original container

Precautionary Statements - Response

IF ON SKIN (or hair): Remove/Take off immediately all contaminated clothing. Rinse skin with water/shower. Take off contaminated clothing and wash it before reuse

IF SWALLOWED: Call a POISON CENTER or doctor/physician if you feel unwell. Rinse Mouth. Do not induce vomiting.

IF INHALED: Remove to fresh air and keep at rest in a position comfortable for breathing. Immediately call a Poison Center or doctor/physician.

IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. If eye irritation persists: Get medical advice/attention.

Absorb spillage to prevent material damage.

Precautionary Statements - Disposal

Dispose of contents/container to an approved waste disposal plant

Precautionary Statements – Storage:

Store Locked Up

Store in corrosive resistant container with a resistant inner liner.

Other Hazards:

Harmful to aquatic life with long lasting effects

**If Chemical Name/CAS No is "proprietary" and/or Weight-% is listed as a range, the specific chemical identity and/or percentage of composition has been withheld as a trade secret.**

First Aid Measures

Eye Contact

Rinse thoroughly with plenty of water for at least 15 minutes, lifting lower and upper eyelids. Consult a physician.

Skin Contact

Wash with soap and water. Take off contaminated clothing. Wash contaminated clothing before reuse. If skin irritation or rash occurs: Get medical advice/attention.

Inhalation

Remove to fresh air.

Ingestion

Clean mouth with water and drink afterwards plenty of water.

Most important symptoms and effects

Symptoms

Contact may cause irritation and redness.

__ 	__ 	__ 	__ 	_

Page  2 / 7

4. FIRST-AID MEASURES

Chemical Name

CAS No

Weight-%

Nonylphenol polyethylene glycol ether

127087-87-0

15-20

Potassium Hydroxide

1310-58-3

10-15

2-Butoxyethanol

111-76-2

10-15

Tetrasodium EDTA

64-02-8

5-10

Sodium Tripolyphosphate

7758-54-9

1-5

Sodium Metasilicate

10213-79-3

1-5

3. COMPOSITION/INFORMATION ON INGREDIENTS

HD  DEGREASER

Revision Date:  7-Jan-2015

__ 	__ 	__ 	__ 	_

Indication of any immediate medical attention and special treatment needed

Notes to Physician

Treat symptomatically.

Suitable Extinguishing Media

Use extinguishing measures that are appropriate to local circumstances and the surrounding environment.

Unsuitable Extinguishing Media  Not determined.

Specific Hazards Arising from the Chemical

Non-flammable solution.

Protective equipment and precautions for firefighters

As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full protective gear.

Personal precautions, protective equipment and emergency procedures

Personal Precautions

Use personal protective equipment as required.

Methods and material for containment and cleaning up

Methods for Containment

Prevent further leakage or spillage if safe to do so.

Methods for Clean-Up

Keep in suitable, closed containers for disposal.

Precautions for safe handling

Advice on Safe Handling

Use personal protection recommended in Section 8. Wash thoroughly after handling. Contaminated work clothing should not be allowed out of the workplace. Avoid breathing vapors or mists.

Conditions for safe storage, including any incompatibilities

Storage Conditions

Store Locked Up

Store in corrosive resistant container with a resistant inner liner.

Keep containers tightly closed in a dry, cool and well-ventilated place.

Incompatible Materials

None known based on information supplied.

Exposure Guidelines

Exposure limits noted for ingredient(s)

__ 	__ 	__ 	__ 	_

Page  3 / 7

8. EXPOSURE CONTROLS/PERSONAL PROTECTION

7. HANDLING AND STORAGE

6. ACCIDENTAL RELEASE MEASURES

5. FIRE-FIGHTING MEASURES

HD  DEGREASER

Revision Date:  7-Jan-2015

__

__

__

__

_

Appropriate engineering controls

Engineering Controls

Apply technical measures to comply with the occupational exposure limits.

Individual protection measures, such as personal protective equipment

Eye/Face Protection

Avoid contact with eyes.

Skin and Body Protection

Wear suitable protective clothing.

Respiratory Protection

Ensure adequate ventilation, especially in confined areas.

General Hygiene Considerations Handle in accordance with good industrial hygiene and safety practice.

Information on basic physical and chemical properties

Physical State Appearance Color

Liquid Clear Blue

Odor

Odor Threshold

Not determined Not determined

Property pH

Melting Point/Freezing Point Boiling Point/Boiling Range Flash Point

Evaporation Rate Flammability (Solid, Gas) Upper Flammability Limits Lower Flammability Limit Vapor Pressure

Vapor Density

Specific Gravity Water Solubility

Solubility in other solvents Partition Coefficient

Auto-ignition Temperature Decomposition Temperature Kinematic Viscosity Dynamic Viscosity Explosive Properties Oxidizing Properties

Values

> 11.5

Not determined

> 212° F

> 250 ° F

< 1

n/a-liquid

Not determined Not determined Not determined Not determined Not determined Complete

Not determined Not determined Not determined Not determined Not determined Water Thin

Not determined Not determined

Remarks  • Method

Reactivity

Not reactive under normal conditions.

__ 	__ 	__ 	__ 	_

Page  4 / 7

10. STABILITY AND REACTIVITY

9. PHYSICAL AND CHEMICAL PROPERTIES

Chemical Name

ACGIH TLV

OSHA PEL

NIOSH IDLH

2-Butoxyethanol   111-76-2

TWA: 25 ppm

TWA: 50 ppm

-

Potassium Hydroxide   1310-58-3

2 mg/m3

2 mg/m3

-

Sodium Metasilicate   6834-92-0

2 mg/m3

2 mg/m3

-

Sodium Tripolyphosphate   7758-29-4

15 mg/m3

15 mg/m3

-

H-40 DEGREASER

Revision Date:  7-Jan-2015

__ 	__ 	__ 	__ 	_

Chemical Stability

Stable under recommended storage conditions.

Possibility of Hazardous Reactions

None under normal processing.

Conditions to Avoid

Keep out of reach of children.

Incompatible Materials

None known based on information supplied.

Hazardous Decomposition Products

None known based on information supplied.

Information on likely routes of exposure

Product Information

Eye Contact

Avoid contact with eyes.  Causes serious eye Damage

Skin Contact

Causes severe skin burns and eye damage

May cause an allergic skin reaction. Causes skin irritation. May be harmful in contact with skin.

Inhalation

Avoid breathing vapors or mists.

Ingestion

Do not taste or swallow.  Harmful if swallowed

Component Information

Information on physical, chemical and toxicological effects

Symptoms

Please see section 4 of this SDS for symptoms.

Delayed and immediate effects as well as chronic effects from short and long-term exposure

Carcinogenicity

Not classifiable as a human carcinogen.

Legend

IARC (International Agency for Research on Cancer)

Group 3 IARC components are "not classifiable as human carcinogens"

__ 	__ 	__ 	__ 	_

Page  5 / 7

Chemical Name

ACGIH

IARC

NTP

OSHA

2-Butoxyethanol    111-76-2

A3

Group 3

Chemical Name

Oral LD50

Dermal LD50

Inhalation LC50

Potassium Hydroxide: 1310-58-3

300 mg/kg (Rat)

-

-

2-Butoxyethanol    111-76-2

= 470 mg/kg  ( Rat )

= 2270 mg/kg  ( Rat ) = 220 mg/kg ( Rabbit )

= 2.21 mg/L  ( Rat ) 4 h = 450 ppm ( Rat ) 4 h

Tetrasodium EDTA   64-02-8

= 10 g/kg  ( Rat )

-

-

Nonylphenol polyethylene glycol ether 127087-87-0

960-6980 mg/kg (Rat)

2000-2991 mg/kg (Rabbit)

1.15 mg/L (Rat) Aerosol, 4 h

Sodium Tripolyphosphate: 7758-29-4

3900 mg/kg (Rat)

4640 mg/kg (Rabbit)

-

Sodium Metasilicate 10213-79-3

847 mg/kg (Rat)

-

-

11. TOXICOLOGICAL INFORMATION

HD DEGREASER

Revision Date:  7-Jan-2015

__ 	__ 	__ 	__ 	_

Numerical measures of toxicity

Calculated ATE (Oral) of this mixture is >1400 mg/kg

Ecotoxicity

Harmful to aquatic life with long lasting effects

Persistence/Degradability

Not expected to be readily biodegradable.

Bioaccumulation

Not determined.

Mobility

Other Adverse Effects

Not determined

Waste Treatment Methods

Disposal of Wastes

Disposal should be in accordance with applicable regional, national and local laws and regulations.

Contaminated Packaging

Disposal should be in accordance with applicable regional, national and local laws and regulations.

Note

Please see current shipping paper for most up to date shipping information, including exemptions and special circumstances.

DOT

UN1814, Potassium hydroxide solution, 8, II, RQ 1000 lbs

__ 	__ 	__ 	__ 	_

Page  6 / 7

14. TRANSPORT INFORMATION

13. DISPOSAL CONSIDERATIONS

Chemical Name

Partition Coefficient

2-Butoxyethanol 	111-76-2

0.81

Chemical Name

Algae/aquatic plants

Fish

Toxicity to microorganisms

Crustacea

Potassium Hydroxide   : 1310-58-3

-

LC50: 100 mg/L

-

-

2-Butoxyethanol 111-76-2

1490: 96 h Lepomis macrochirus mg/L LC50 static 2950: 96 h Lepomis macrochirus mg/L LC50

1698 - 1940: 24 h Daphnia

magna mg/L EC50 1000: 48 h Daphnia magna mg/L EC50

Tetrasodium EDTA 64-02-8

1.01: 72 h Desmodesmus subspicatus mg/L EC50

41: 96 h Lepomis macrochirus mg/L LC50 static 59.8: 96 h Pimephales promelas mg/L LC50 static

610: 24 h Daphnia magna mg/L EC50

Sodium Tripolyphosphate 7758-29-4

-

-

-

EC50: 276.6 mg/L; Daphnia’ 48h

Nonylphenol, branched, ethoxylated 127087-87-0

-

LC50: 3.8-6.2 mg/L; fathead minnow, 96h

IC50: >1000 mg/L,

Baacteria, 16h

LC50: 9.3-21.4 mg/L; Water flea, 48h

12. ECOLOGICAL INFORMATION

HD  DEGREASER

Revision Date:  7-Jan-2015

__

__

__

__

_

International Inventories

Not determined

US Federal Regulations

Cercla:

SARA 313

This product contains the following toxic chemicals subject to the reporting requirements of section 313 of the Emergency Planning and Community Right-To-Know Act of 1986 (40 CFR 372).

US State Regulations

U.S. State Right-to-Know Regulations

Not determined

Issue Date: Revision Date: Revision Note:

7-Jan-2015

7-Jan-2015

New format

Disclaimer

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at the date of its publication. The information given is designed only as a guidance for safe handling, use, processing, storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The information relates only to the specific material designated and may not be valid for such material used in combination with any other materials or in any process, unless specified in the text.

End of Safety Data Sheet

__ 	__ 	__ 	__ 	_

Page  7 / 7

16. OTHER INFORMATION

Chemical Name

New Jersey

Massachusetts

Pennsylvania

Potassium Hydroxide

X

X

X

Nonylphenol, branched, ethoxylated

X

X

X

2-Butoxyethanol

X

X

X

Sodium Metasilicate

X

-

X

Sodium Tripolyphosphate

X

X

X

Chemical Name

CAS No

Weight-%

SARA 313 - Threshold Values %

2-Butoxyethanol

111-76-2

10-15

1.0

Chemical Name

Hazardous Substances RQs

CERCLA/SARA RQ

Reportable Quantity (RQ)

Sodium Tripolyphosphate 7758-29-4

5000 lbs

5000 lbs

15. REGULATORY INFORMATION