NPCA Hazardous Materials

Identification System

Specific HazardPersonal Protection

4-SevereX-Ask a supervisor

3-Serious A-Safety Glasses

2-ModerateB-Goggles, Gloves

1-SlightC-Goggles, Gloves, Apron

0-Minimal D-Goggles, Gloves, Apron, &

Faceshield

Health (2)Reactivity (0)

Flammability (0)Personal Protection

Safety Data Sheet

May be used to comply with OSHA’s

Hazardous Communication Standard, 29 CFR 1910.1200

IDENTITY (As used on LABELS and LIST)

MIGHTY Green Degreaser

SECTION I

MFGR’S NAME AND ADDRESS		SUNLINE L.L.C.

315 Brookhaven Dr  -  ORLANDO, -  FL  32803-1895

EMERGENCY TELEPHONE NUMBER		407-415-8751		DATE PREPARED:

Telephone Number For Information		407-896-2129 	                    9-2-15

SECTION II - HAZARDS IDENTIFICATION

CLASSIFICATION

Hazard Statements

H303- May be harmful if swallowed

H315- Causes skin irritation

H320- Causes eye irritation.

Signal Word  WARNING

Precautionary Statements – Prevention

Do not breathe vapor, mist, or spray

Wash face, hands and exposed skin after use

Wear protective gloves, clothing and eye protection

Precautionary Statements – Response

EYES:  Rinse eyes with water for at least 15 minutes, lifting lids.  Remove contact lenses if present and possible. Call physician immediately.

SKIN: Remove contaminated clothing

Rinse skin with water/shower. Launder clothing before re-use.

INHALATION:  Remove victim to fresh air and keep at rest in comfortable position. Call physician immediately.

INGESTION:  Rinse mouth.  Do not induce vomiting.

________________________________________________________________________________________

Page 1 of 6

Safety Data Sheet                                       Mighty Green Degreaser                               9-02-15

SECTION III -  INFORMATION ON INGREDIENTS

Specific Components	          Wt %

Sodium Hydroxide                        5

CAS#1310-73-2

Sodium Metasilicate 	              3

CAS #6834-92-0

SECTION IV  FIRST AID MEASURES

EYE:  Rinse with water for at least 15 minutes, lifting lids.  Remove contact lenses if present and possible.  Seek medical attention immediately.

SKIN:  Remove contaminated clothing.  Wash skin with water /shower.  Wash contaminated clothing before re-use.  Seek medical attention if irritation develops or persists.

INHALATION:  Remove victim to fresh air.  Keep at rest in comfortable position.  Seek medical attention immediately.

INGESTION: Rinse mouth with water. Give  milk or water to dilute.  Never give anything by mouth to an unconscious person.  Neutralize with 2 tablespoons of vinegar. Do not induce vomiting.  Seek medical attention immediately.

Note To Physician:  Treat symptomatically.

SECTION V  FIRE FIGHTING MEASURES

Suitable Extinguishing Media – Water, Foam, Dry Chemical

Specific Hazards – NA

Protective Equipment and Precautions for Firefighters – Always wear full protective gear and self-contained breathing apparatus.

SECTION VI   ACCIDENTAL RELEASE MEASURES

Personal Precautions, Protective Equipment and Emergency Procedures

Personal Precautions – Use personal protective equipment as required.

Methods and Materials for Clean-up

Containment – Prevent further spillage or leakage if safe to do so.

Clean-Up – Collect with sand, clay or other absorbent .  Sweep up and place in suitable container for disposal.

Page 2 of 6

Safety Data Sheet                                  Mighty Green Degreaser                                          9-02-015

SECTION VII   HANDLING and STORAGE

Precaution for Safe Handling – Wash thoroughly after handling or product use.  Use personal protective equipment as recommended in Section VIII.  Do not breathe gas, mist vapor or spray.

Storage Conditions – Keep containers tightly closed in as cool, dry well ventilated area.

Incompatible Materials – Strong acids.

SECTION VIII   EXPOSURE CONTROLS – PESONAL PROTECTION

Exposure Guidelines

ACGIH TLV              OSHE PEL            NIOSH IDLH

Sodium Hydroxide CAS#  1310-73-2      Ceiling: 2mg/m3      TWA: 2 mg/m3       IDLH: 10 mg/m3

Engineering Controls – Local exhaust ventilation recommended.

Individual Protection Measures/Equipment

Eye/Face Protection:  Wear eye/face protection.

Skin/Body Protection:  Wear suitable gloves.

Respiratory Protection:  Ensure adequate ventilation.

General:  Handle in accordance with good industrial hygiene and safety practice

SECTION IX   PHYSICAL and CHEMICAL PROPERTIES

Physical State:  Liquid.   Appearance:  Green    Odor:  Little to no odor.

BOILING POINT (F)		Over 2120F			SPECIFIC GRAVITY (H2O=1)	1.0156

VAPOR PRESSURE (mmHg)	N/A				MELTING POINT		N/A

VAPOR DENSITY (air=1)	N/A				EVAPORATION POINT	Less than 1

SOLUBILITY IN WATER	Complete			(Butyl Acetate=1)

APPEARANCE AND ODOR:  Clear, green liquid, water-thin liquid with little odor.

SECTION X -   STABILITY and REACTIVITY

Reactivity – Not reactive under normal conditions

Chemical Stability – Stable under recommended storage conditions.

Hazardous Reactions – None under normal processing

Hazardous Polymerization – does not occur.

Conditions To Avoid – Keep out of reach of children.

Page 3 of 6

Safety Data Sheet                                        Mighty Green Degreaser                              9-02-2015

Incompatible Materials – Acids

Hazardous Decomposition Products – None known

________________________________________________________________________________________________

SECTION XI – TOXICOLOGICAL INFORMATION

Information on likely routes of exposure

Product Information

EYE:     Causes severe eye damage

SKIN:   Causes severe skin burns

INHALATION:  Avoid breathing vapor or mist

INGESTION:  May be harmful if swallowed

Component Information

Chemical Name                        Oral LD 50                          Dermal LD50                         Inhalation LC50

Sodium Hydroxide                          X                                = 1350 mg/kg (rabbit)                       X

CAS 1310-73-2

Carcinogenicity – This product does not contain anu carcinogens or potential carcinogens as listed by OSHA.

SECTION XII -  ECOLOGICAL INFORMATION

Ecotoxicity

Harmful to aquatic life with long lasting effects.

Component Information

Chemical                               Algae/ Aquatic Plants                     Fish

Sodium Hydroxide                              X                                         45.4:96 Hr Oncorhynchus

1710-73-2                                                                                      mykiss mg/L  LC50 static

SECTION XII – DISPOSAL CONSIDERATIONS

Waste Treatment Methods

Disposal -  Disposal should be in accordance with all applicable Federal, state and local regulations  Do not re-use empty containers.

Page 4 of 6

Safety Data Sheet                                           Mighty Green Degreaser                                                  9-02-2015

SECTION XIV – TRANSPORT INFORMATION

Note:  Please see current shipping paper for most up to date shipping information including exemptions and special circumstances.

DOT

UN ID#                                       UN 3267

Proper Shipping                       Name  Corrosive liquid, basic, inorganic, n.o.s. (Sodium hydroxide)

Hazard Class                            8

Packing Group                         II

IATA                                         See Above

IMDG                                       See above

Marine Pollutant                     This material may meet the definition of a marine pollutant

SECTION XV – REGULATORY INFORMATION

US Federal Regulations

CERCLA

This material, as supplied, contains one or more substances regulated as a hazardous substance under the Comprehensive Environmental Response Compensation and Liability Act (CERCLA) (40 CFR 302)

Chemical Name                        Hazardous Substance RQ’s                        Reportable Quantity

Sodium Hydroxide                                1000 lbs                                              RQ 1000 lbs in final RQ

1310-73-2                                                                                                        RQ 454 kg in final RQ

Sara 313

This product does not contain any chemicals which are subject to the reporting requirements of the Act and Title 40 of the Code of Federal Regulations, Part 372

CWA (Clean Water Act)

This product contains the following substance(s) which are regulated pollutants pursuant to the Clean Water Act (40 CFR 122.21)

Component      CWA Reportable     CWA-Toxic Pollutants   CWA Priority Pollutants      CWA Hazardous

Quantities                                                                                                    Substances

Sodium Hydroxide     1000 lbs                                                                                                        X

CAS 1310-73-2

US State Right to Know Regulations

Chemical                             New Jersey            Massachusetts           Pennsylvania

Sodium Hydroxide                        X                              X                                 X

CAS 1310-73-2

Page 5 of 6

Safety Data Sheet                               Mighty Green Degreaser                                             9-2-2015

SECTION XVI – OTHER INFORMATION

NFPA         Health Hazards          Flammability            Instability               Special Hazards

Not determined         Not determined        Not determined          Not determined

HMIS     Health Hazards          Flammability           Physical Hazards        Personal Protection

2                              0                                  1                           Not determined

Disclaimer

The information provided in this safety data sheet is correct to the best of our knowledge, information and belief at the date of issue.  The information is designed only as a guidance for safe handling, use processing, storage transportation, disposal and release and is not to be considered a warranty or quality specification.  The information relates only to the specific material designated  and may not be valid for such material used in combination with any other materials or in any process, unless specified in the text.

Page 6 of 6